4能互补能有效降低弃电率:以舟山群岛为例

- Abstract

Introduction

随着全球经济的持续增长和能源消耗的不断攀升，传统化石能源的发电模式已难以持续支撑社会发展的长期需求，推动新能源开发与利用成为全球能源转型的关键议题。在此背景下，占据地球表面积约71%的海洋，以其巨大的可再生能源储量逐渐成为重要的研究焦点[1][2]。然而，海洋能，尤其是海浪能和风能，具有显著的季节性和随机波动性。因此单独开发一种能源会因其巨大的波动性[3]，而受到制约。为应对单一能源的波动性挑战，多能互补系统（Multi-Energy Complementary System, MEC）的概念被提出[4]，并被证实是提高能源供给稳定性和经济效益的有效路径。早期研究已证实了风光互补[5]和多种可再生能源组合[6]的优越性。早期的相关研究多聚焦于陆地能源，研究者将目光转向海洋则是在过去的十年内开始的。来自加那利群岛技术研究所研究展示了风-水蓄结合如何在实际岛屿系统中显著提升可再生覆盖率[7]。通过优化可再生能源与储能的安装容量配比，可以显著降低输出波动性以及减少储能需求与弃电率[8]（Pearre，2020）。并且随着研究的深入，越来越多的指标被引入用于评估海上多能互补系统的科学性、可靠性[9][11][13]。

With the continuous rise of the global economy and increased energy consumption, existing fossil fuel-based power generating methods can no longer sustainably fulfill society's long-term development demands. This has made the development and utilization of new energy sources a major issue in the global energy transition. In this context, the oceans—covering over 71% of the Earth's surface—have become a major study target owing to their immense renewable energy potential [1][2]. However, ocean energy, particularly wave and wind energy, displays large seasonal and random changes. As a consequence, establishing a single energy source is constrained by its great variability [3]. To tackle this difficulty, the idea of Multi-Energy Complementary Systems (MEC) was created [4], showing to be an efficient solution to increase energy supply stability and economic efficiency. Early studies have proven the benefits of wind-solar complementary systems [5] and combinations of different renewable energy sources [6]. Although earlier studies mostly focussed on land-based energy sources, interest in marine systems has expanded during the last decade. Research from the Instituto Tecnológico de Canarias (ITC) has showed how wind-water storage combination may considerably boost renewable energy coverage in island systems [7]. By optimizing the capacity ratio between renewable production and energy storage, swings in output may be considerably decreased, along with lower storage demands and curtailment rates [8] (Pearre, 2020). Additionally, as research continues, new criteria have been created to measure the scientific rigor and dependability of offshore multi-energy complementary systems [9][11][13].

在过去的数十年当中，全世界范围内已开展多次海洋能探索试点，受制于过去的技术原因，部分试点最终以失败告终。而失败的原因可分为技术问题，例如2008年葡萄牙的Aguçadoura Wave Farm，仅运行数月就因为轴承故障等问题停运；商业问题如2010年开始的英国/葡萄牙测试与示范项目Pelamis Wave Energy，最终因为母公司财务问题，导致停运；许可问题如2020年美国的Roosevelt Island Tidal Energy因项目许可到期后已按计划拆除并完成退役等。因此以上出现的问题可供我们后续进行多能互补实验时作为参考，并且检验已开展的试点是否可以长久运行。在评估时，不仅要考虑能源利用效率[12]、发电量、商业成本和弃电率[14]等多种因素。

Over the last few decades, various maritime energy exploration pilot projects have been established globally. Constrained by prior technology restrictions, several of these attempts eventually ended in failure. The causes of failure can be categorized into technical issues—such as Portugal's AguçadouraWave Farm, which shut down after only a few months of operation in 2008 due to bearing failures and other problems— commercial issues, like the UK/Portugal Pelamis Wave Energy test and demonstration project launched in 2010, which ultimately ceased operations due to financial difficulties at its parent company; and permitting issues, such as the Roosevelt Island Tidal Energy project in the United States in 2020, which was dismantled and decommissioned as planned after its project permit expired. Therefore, these prior problems may serve as helpful references for our forthcoming multi-energy complementary studies and assist assess whether current pilot projects can attain long-term operation. During assessment, various variables must be addressed, including energy consumption efficiency [12], power output, commercial expenses, and curtailment rates [14].

为了评估不同能源组合的利用效率，通常引入容量因子（Capacity Factor, CF），CF因其能够量化发电设备的实际出力与最大潜能之比，成为评估多能互补系统性能的关键指标[11][13]（Stoutenburg，2010；Khan，2022）。同时，研究者们通过最小化综合容量因子的标准差（Std Dev）来追求系统的稳定性[15]（Vázquez，2024），并引入遗传算法（GA）、蚁群算法（BOACA）等启发式算法来寻找最优配比方案[17][18]（Sawle，2017；Abo-Elyousr，2018）。

然而，上述常用的评估指标和优化算法仍存在亟待解决的局限性。一方面，以CF或Std Dev等统计参数为目标的优化方法，在追求稳定性的同时，往往因配比方案的过度集中而牺牲了系统的整体平均出力和能源利用效率。另一方面，传统的权重确定方法易受主观经验影响，缺乏客观量化依据。启发式算法虽然能够求解复杂问题，但在系统初筛和快速决策阶段，又存在计算复杂度高和求解精度之间的矛盾。因此，本研究的核心问题在于，如何在多能互补系统中，构建一个客观、动态的配比决策机制，以有效平衡能源的利用率（高CFt）与系统稳定性（低Std Dev）。上述这些常用的评估指标和优化算法仍存在诸多局限性，这些局限性是未来研究亟需突破的关键。比如以CF、标准差等统计参数来评估系统性能的方法，在快速筛选和初步分析阶段具有效率高、易于量化的优点，但在实际系统运行的精细化评估中，AHP存在着过于主观，其他算法又过于将目光聚焦某一单一参数。而GA、BOACA等又存在着计算复杂度和求解精度之间的矛盾。

近年来，随着客观赋权思想被引入能源系统综合评价与方案优选，其中熵权法能够依据指标的信息量与变异度自适应确定权重，减少主观性并提升可解释性。使用熵权法作为核心权重计算方法，可用于提升离网混合能源系统多目标优化设计中决策的客观性[19]。熵权法还可以和其他评估方法连接起来，避免评估结果的主观臆断，又不脱离实际应用需求[20]，并且可以用于多能互补生态系统评价模型中，以确定评价指标的客观权重[21]。因此在本研究中我们引入了熵权法用于评估各能源在本系统中的占比。熵权法能够根据各能源容量系数时间序列的变异程度，客观地赋予权重，从而克服传统赋权法的主观性，并实现系统利用率与稳定性的最优权衡。

本研究综合考虑了系统可行性、经济性等关键指标，并以中国舟山群岛海域的风能、波浪能、潮流能和太阳能四种能源的混合系统为案例，验证所提优化方法的有效性。此外，本文将引入弃电量和捕获价格进行敏感性分析，评估不同配比方案的经济可行性和价格自噬抑制能力。

To analyze the utilization efficiency of various energy combinations, the capacity factor (CF) is typically proposed. CF serves as a significant indication for assessing multi-energy complementary system performance, as it measures the ratio of actual output to maximum theoretical capacity of power production equipment [11][13] (Stoutenburg, 2010; Khan, 2022). Simultaneously, researchers target system stability by decreasing the standard deviation (Std Dev) of the composite capacity factor [15] (Vázquez, 2024), and apply heuristic algorithms such as genetic algorithms (GA) and ant colony optimization algorithms (BOACA) to determine optimum allocation schemes [17] [18] (Sawle, 2017; Abo-Elyousr, 2018). 
However, the generally used assessment criteria and optimization techniques outlined above still have drawbacks that need to be addressed. On one side, optimization strategies targeting statistical metrics such as CF or Std Dev sometimes forfeit the system's overall average power output and energy usage efficiency owing to excessive concentration of the design scheme while seeking stability. On the other hand, conventional weight estimation techniques are vulnerable to subjective experience and lack objective mathematical foundation. While heuristic algorithms may tackle complicated issues, they suffer a trade-off between computing complexity and solution accuracy during early system screening and quick decision-making stages. Therefore, the key topic of this work is how to build an objective, dynamic allocation decision mechanism in multi-energy complementary systems to successfully balance energy usage (high CFt) and system stability (low StdDev). These extensively used assessment measures and optimization algorithms nevertheless reveal several shortcomings, highlighting crucial areas needing breakthroughs in future research. For instance, approaches measuring system performance using statistical measures like CF and standard deviation provide great efficiency and simplicity of quantification during quick screening and preliminary analysis. However, in the comprehensive examination of real system functioning, AHP suffers from excessive subjectivity, while other algorithms tend to concentrate too narrowly on a single parameter. Meanwhile, algorithms like GAs and BOACA confront trade-offs between computational complexity and solution accuracy.
In recent years, the notion of objective weighing has been brought into the entire assessment and optimization of energy systems. Among these techniques, the Entropy Weighting Method (EWM) may adaptively estimate weights depending on the information content and variability of indicators, decreasing subjectivity and boosting interpretability. Employing the entropy weight technique as the core weight calculation strategy may increase the objectivity of decision-making in multi-objective optimization design for off-grid hybrid energy systems [19]. This approach may also be linked with other evaluation methodologies to reduce subjective bias in assessment conclusions while maintaining aligned with practical application demands [20]. Furthermore, it may be employed in multi-energy complementary ecosystem assessment models to derive objective weights for evaluation indicators [21]. Therefore, this research presents the entropy weight approach to analyze the percentage of each energy source inside the system. By objectively assigning weights based on the unpredictability of time series data for each energy source's capacity coefficient, the entropy weight technique overcomes the subjectivity of previous weighting methods and achieves an ideal balance between system utilization and stability. 

This research extensively evaluates critical variables such as system feasibility and economic viability. Using a hybrid system incorporating wind, wave, tidal, and solar energy from China's Zhoushan Archipelago waters as a case study, we verify the efficacy of the suggested optimization strategy. Additionally, this article offers sensitivity analysis utilizing curtailed power volume and capture price to analyze the economic feasibility and price self-cannibalization suppression capacities of alternative allocation methods.

Data and Methods

本章旨在详细阐述本研究采用的资源数据、研究区域的特征以及多能互补系统配比优化的核心方法。研究的科学性和可靠性建立在精确的数据输入和严谨的模型基础之上，以确保配比结果的科学性。

Study area

本研究区域为舟山群岛邻近海域，经纬度范围为纬度 29.4°–31.0°N、经度121.5°–123.5°E，如Figure 1所示。舟山群岛位于中国东海，是中国最大的群岛，占全国岛屿总数的20%，拥有1390个岛屿，人口达100万[22]。该区域以复杂的海岸带和群岛地貌格局为特征，拥有强劲且稳定的风力资源[23]；受西北太平洋影响，具备较高的涌浪驱动波浪能潜力[24]；并且舟山群岛富含丰富的潮流能资源，大潮功率密度峰值超过25000kW/m2，流向稳定系数大于0.95[25]；年日照时长平均超过2000小时，太阳辐射充沛。这种优异的资源禀赋为风能、波浪能、潮流能和太阳能的协同开发创造了有利条件，使得该区域在海上能源开发方面极具前景。

Figure 1

中国政府在舟山群岛区域已率先开展了一系列多能互补集成优化项目，旨在探索和应用风能、太阳能、潮流能及天然气等多种能源的协调开发与高效利用。在秀山岛兆瓦级潮流能项目，如S5，和东极岛自愈式海岛智慧微电网，如S6，成功实施了潮流能与光伏能的结合。这些项目成功实现了对海岛及周边海域的持续、稳定电力供应，有效缓解了海岛居民的用电短缺问题，显著提升了海岛能源安全保障能力。并且这些站点大多数仍在更新升级技术阶段，用于海上长期试验与装备测试。

鉴于此前的项目试点成果与经验积累，本研究旨在对舟山群岛的多能互补开发潜力区域进行系统性划分。尽管此前鲜有公开文献专门针对舟山群岛的多能互补问题提出区域划分方案，但基于对既往项目选址的地理集中度、能源禀赋分布差异及区域电网结构的综合分析，本研究将舟山群岛区域科学地划分为四个高潜力开发区域（Area 1, Area 2, Area 3, Area 4）。Figure 1右清晰展示了部分既有项目选址的地理位置以及本研究划分的四个重点分区，这为后续多能互补系统的优化配置与推广应用提供了合理且具有实践基础的地理研究框架。

Data

基础资源评估数据以欧洲天气中心ERA5再分析数据集为核心，辅以中国东海波浪观测站实测波参数、舟山岱山潮位测站实测潮位流速数据及NOAA、GEBCO全球地形数据[26]；然后通过SWAN模型与MIKE模型生成模拟数据，且经实测数据验证；同时调研国内外各风机、波浪能转换器、潮流能装置及光伏板的厂商技术手册参数[27][28]，参考国家/行业标准标准，所有数据均经模型验证与标准化处理以保障可靠性。

Capacity factors

容量系数（CF）是衡量发电厂能源输出效率的重要指标，其定义为发电厂的实际总能源输出量与该发电厂以满容量运行时理论上的最大发电量之比，通过整理先前整理的数据库以及参考其他文献可得出CF如式所示：

式中Pt代表平均功率即实际发电量与时间之比，Pe代表装机容量即电站或设备在设计上能够连续稳定输出的最大功率[29]。

CFwind

风能容量系数可参考Wen等人[30]的做法，先对一段时间内（如1年）的瞬时功率Pt取平均，再与风机以额定功率Pe持续运行一年的理论发电量取比值，即可得到

式中ρ为空气密度，本文取值1.225kg/m3；Cp为风能利用系数，现代风机取值为0.4；AT为风机叶片扫风面积，本文使用的风机叶片直径为154m；V为风机hub高度的实际风速，本文取100m。

CFwave

波浪能容量系数可参考Bahareh Kamranzad 等人的做法[31]，使用WEC在特定站点的实际年/月发电量与WEC的理论最大能量产出取比值，如式：

CFtidal

如式[32]参照风机功率曲线模型，将“风速”替换为“潮流流速（v）”，“空气密度”替换为“海水密度（ρwater=1025kg/m3）。

式中AT为潮流能水轮机扫流面积；Ptidal为潮流能转换器额定功率；Cp为水轮机功率系数（由额定参数推算，同风能逻辑）。

CFsolar

太阳能CF需结合光伏板转换效率、实际太阳辐射强度（考虑温度、风速对光伏板性能的影响），先计算实际平均发电功率，再与光伏板额定功率对比，引入“性能比PR”修正环境因素影响。如式所示[33]

式中RSDS式太阳能辐射密度，STC是参考测试情况（RSDSSTC=1000W/m2）。

Complementary analysis

确定多能互补系统中最优的能源配比是提升系统稳定性和利用效率的关键挑战。传统的配比优化方法存在多重局限：例如，它们往往依赖历史数据确定固定权重，导致其无法实时响应风能容量系数的季节波动 ；权重分配依赖经验判断，缺乏客观量化依据，影响配比方案的科学性；此外，阈值适配性不足也易导致优化流程繁琐且效率低下。

在追求系统稳定性的传统优化路径中，一种直观的方法是采用最小标准差法进行配比，即以CFt的std最小化作为唯一目标。然而，这种方法往往过度强调稳定性，可能导致高能源利用率的资源被低估，最终可能牺牲整体系统的平均出力和经济效益。

为克服传统赋权方法中存在的权重静态化、主观性等局限，并避免单纯追求最低波动性而带来的利用率损失，本研究旨在构建一个客观、动态、基于资源变异性的赋权模型。为此，我们引入了改进的熵权法（The Entropy Weight Method, EWM）作为计算方法。熵权法能够根据各指标（即各能源的容量系数时间序列）的变异程度，利用信息熵客观地计算各指标的熵权，从而得到较为客观的指标权重。

值得注意的是，在仅通过熵的定义进行计算时存在一定误差，当信息熵值越靠近1时，信息熵值很小的波动就会使熵权的结果发生成倍数的变化，造成计算结果不合理。为此，本文中利用改进后的熵权法对指标进行客观赋权。具体如下。

（1）根据拟定的方案数据，建立原始信息矩阵为

（2）指标归一化处理，得到标准化矩阵R的元素rij为

式中：1⩽i⩽m；1⩽j⩽n。

（3）计算指标信息熵Ej为

（4）计算权重ωj为

在计算出各能源在多能互补中的占比之后，我们通过式衡量各能源组合能源利用效率。

其中wi是第i个能源的熵权，CFt是对应的容量系数。

标准差计算公式如下：

其中N是数据点数量，xi是每个数据点，μ是数据平均值。

Sensitivity analysis

为评估舟山群岛各多能互补系统配比方案的可行性以及抗风险能力，本研究引入弃电量和捕获价格作为敏感性指标，对不同区域的开发潜力进行深入剖析，以提供更具经济可行性的开发建议。

弃电量是指系统发出的电量中，由于电网限制、输电容量不足或瞬时出力超过当地负荷需求而未能被利用的电量。其计算公式为

其中Pgen,t指系统在t时刻的总发电功率，计算公式为

Pload,t指系统在t时刻的实际负荷需求或电网可接纳容量，是决定了是否弃电的关键约束根据Zhang等人(2018)[35]的研究，由于中国北麂岛的人口密度和舟山群岛人口密度相近，因此我们以峰值负荷800kW作为参考，Husein等人(2017)[36]和Bertheau等人(2020)[37]的研究表明，小型海岛，以低能耗产业为主（渔业，农业等）的平均负荷约等于峰值负荷的0.5倍左右，因此舟山群岛的平均负荷可认为是400kW；同时采用用电早高峰和晚高峰的动态负荷计算方式，因此选取正弦函数生成动态负荷作为理想化的负荷；Δt为时间步长，通常为1小时。

捕获价格是可再生能源发电的实际加权平均售价。如果Pcap高于市场平均电价，说明该系统出力与高电价时段（如晚高峰）匹配良好，经济效益高。

其中Pdelivered,t代表系统在t时刻实际输送到电网或负荷功率，计算公式如下

其中Pcur,t是t时刻的弃电功率。

Pricet表示系统在t时刻的电能市场价格，根据2024年度中国电力市场发展报告[38]研究表明，中国平均市场化交易价格在0.4元每千瓦时，ESold指系统在全年实际总发电量。

Results

本研究基于容量因子（CFt）分析了舟山群岛区域内多种可再生能源的年内变化规律及其互补特性。CFt 及其标准差（σ）用于衡量能源利用率与输出波动性，前者反映装机产能利用水平，后者反映系统的平稳性。理想的多能系统应在保证较高 CFt 的同时保持较低的σ，从而实现稳定高效的能源供给。

Temporal Characteristics of Capacity Factors for Each Energy Source

四类能源的年内变化如Figure 2所示，表现出显著的季节差异。风能在冬季（12月至翌年2月）达到峰值，夏季出力显著下降；波浪能的时序特征与风能高度一致，在冬季随风场增强而上升。太阳能则在夏季（6月至8月）达到最大值，冬季辐照明显减弱，与风能呈典型的反相关系。潮流能受天文规律控制，全年输出较为稳定，季节性波动最小。季度箱线图（Figure 3）进一步揭示出风能的波动幅度最大，而潮流能的离散性最低。总体而言，舟山海域呈现“风波同峰、光伏反相”的时间格局，为实现多能互补提供了天然的资源基础。

Figure 2

Figure 3

能源间的时序互补性

不同能源间的时序互补性通过相关系数进行量化（Figure 4）。风能与太阳能之间的相关系数为 −0.8074（p < 0.01），呈显著负相关，表明两者在季节尺度上具有较强的互补性。风能与波浪能的正相关系数为 0.5365，说明二者受相似的气候驱动机制共同影响。潮流能与其他能源的相关性接近零，表现出稳定而独立的出力特征。综合分析可知，冬季风能与波浪能的高出力能够有效弥补光伏低产期的不足，而夏季光伏增强则抵消风波衰减带来的波动，从而在年尺度上实现出力平衡。

Figure 4

Estimates of Energy Weight Allocation

基于熵权法确定的指标权重，进一步计算了各分区的能源占比与综合性能（Table 1）。结果显示，舟山群岛整体以潮流能（31.59%）和太阳能（44.68%）为主导，系统综合 CFt 为 0.1495，标准差为 0.1114。各分区差异明显：Area 1 受波浪能影响较大（62.64%），但 CFt 与稳态表现均较差（0.0373 与 0.0357）；Area 2 和 Area 3 以潮流能与太阳能为主，CFt 稳定在 0.145–0.153 之间；Area 4 波浪能占比较高（34.93%），整体性能居中。

Table 1

| Area | Wind | Wave | Tide | Solar | CFt | std |
| --- | --- | --- | --- | --- | --- | --- |
| Total | 14.84% | 8.90% | 31.59% | 44.68% | 0.1495 | 0.1114 |
| Area 1 | 9.10% | 62.64% | 14.56% | 13.70% | 0.0373 | 0.0357 |
| Area 2 | 16.06% | 8.82% | 34.44% | 40.68% | 0.1454 | 0.1085 |
| Area 3 | 15.90% | 11.04% | 33.18% | 39.88% | 0.1526 | 0.1097 |
| Area 4 | 12.00% | 34.93% | 24.59% | 28.49% | 0.1188 | 0.0850 |

为了对比熵权法与传统方法之间的优缺点，我们引入通过最小标准差法计算的各能源权重。相比之下，最小标准差法（Table 2）虽能显著降低波动性，但易导致权重分配极端、系统利用率下降。熵权法则通过信息熵度量指标差异，实现了稳定性与效率的平衡，表现出更合理的工程适用性。

Table 2

| Area | Wind | Wave | Tide | Solar | CFt | std |
| --- | --- | --- | --- | --- | --- | --- |
| Area 1 | 0.00171190% | 99.90% | 0.07% | 0.03% | 0.0006 | 0.0006 |
| Area 2 | 0.00000083% | 79.96% | 18.13% | 1.91% | 0.0479 | 0.0327 |
| Area 3 | 0.00000002% | 84.20% | 13.17% | 2.63% | 0.0464 | 0.0370 |
| Area 4 | 0.00000603% | 97.71% | 1.31% | 0.98% | 0.0181 | 0.0177 |

最小标准差法为追求最低波动性，其权重分配呈现极端集中的趋势。例如，在Area 1中，权重几乎完全分配给波浪能，而风能、太阳能和潮流能的占比极低。相似地，Area 4的配比过度依赖波浪能（97.71%）。相比之下，熵权法的配比更为均衡，尤其是在Area 2、Area 3行，均是以波浪能为主力，符合舟山群岛整体资源禀赋的天然互补特性。

稳定性与利用率的权衡方面，最小标准差法在Area 1和Area 4取得了最低的std，远远低于熵权法。这体现了其对稳定性的绝对优化。然而，最小标准差法在实现低std的同时，也往往导致CFt显著降低。例如，Area 1的CFt仅为0.0006，远低于熵权法优化结果。这印证了我们先前的判断：单纯以最小std为目标，可能因配比过于集中，牺牲了系统的平均出力和能源利用效率。

并且从Table 1中可以看出，舟山群岛整体上以太阳能和潮流能为主要能源作为开发对象，风能和波浪能占比相对较低，虽然太阳能的容量系数在冬季最低，但其波动性与风能的季节天然负相关性肯定了太阳能作为多能互补系统主要组成部分的潜力，这是对试点项目开发的有力支撑。同时，潮流能因其全年稳定的高CF水准以及波动较小的特性，使其出现信息熵低而权重高的优点，是系统稳定运行的重要保障。波浪能CF与风能季节性趋势基本一致，在秋冬季较高，夏季较低，波动性较大，因此被赋予最低权重。

根据熵权法得到的客观权重，可以构建一个以太阳能和潮流能为主力，风能和波浪能为辅的配比方案，以期达到利用率和稳定性的优化平衡。该客观赋权方法有效避免了传统配比方法中权重确定的主观性问题。

典型试点站点（S1–S4）的评价结果（Table 3）进一步验证了区域差异性。各站点的 CFt 介于 0.09–0.11 之间，标准差均低于区域平均值。S1 与 S4 受潮流能贡献显著，S3 波浪能占优，S2 光伏比例较高。总体上，站点级分析与区域结果一致，说明以潮流能与太阳能为核心、风能与波能为辅助的组合方案更有利于系统稳定运行。

Table 3

| Area | Wind | Wave | Tide | Solar | CFt | std |
| --- | --- | --- | --- | --- | --- | --- |
| S1 | 13.20% | 0.00% | 56.70% | 30.11% | 0.1035 | 0.0821 |
| S2 | 12.06% | 13.17% | 44.70% | 30.07% | 0.1115 | 0.0837 |
| S3 | 9.25% | 49.37% | 19.66% | 21.72% | 0.0882 | 0.0636 |
| S4 | 9.81% | 4.95% | 58.03% | 27.21% | 0.0997 | 0.0724 |

Performance Evaluation and Regional Strategies

结合功率输出与经济性指标（Table 4）进一步分析，Area 4 的平均功率与总发电量最高，但也面临较高的弃电风险和较低的捕获电价，表明其资源开发需配套储能或灵活负荷（如制氢、海水淡化等）以吸收波动出力[41]。Area 2 与 Area 3 在稳定性与发电效率间取得较好平衡，适合作为并网开发的优先区域；而 Area 1 虽波动性最小，但出力较低，不宜作为主要开发区。

Table 4

| Area | PRated(kW) | ETotal(MWh) | PAvg(kW) |
| --- | --- | --- | --- |
| Total | 1218.7 | 1600.3 | 182.19 |
| Area 1 | 4105.1 | 1345.1 | 153.13 |
| Area 2 | 1271 | 1623.3 | 184.8 |
| Area 3 | 1391.8 | 1865.4 | 212.37 |
| Area 4 | 2617.6 | 2732.4 | 311.07 |

Sensitivity Analysis

为进一步评估经济影响，对弃电量与捕获电价进行了敏感性分析（Table 5）。结果显示，Area 4 弃电率最高（21.52%），捕获电价最低（0.3186 元/kWh），反映供给过剩导致的市场折价；Area 2 弃电率最低（6.08%），捕获电价最高（0.3733 元/kWh），体现其优良的负荷匹配特性。经熵权法优化后，系统整体价格折损率由 20.35% 降至 6.67%，显著提升了经济收益。这表明多能互补配置不仅能增强系统运行稳定性，还能有效缓解可再生能源在市场中的价格自蚀效应。

Table 5

| Area | ECurtailed (MWh) | Curtailment Rate | ESold (MWh) | Pcap(Yuan/kWh) |
| --- | --- | --- | --- | --- |
| Total | 96.88 | 6.05% | 1503.5 | 0.3729 |
| Area 1 | 95.15 | 7.07% | 1250.0 | 0.3693 |
| Area 2 | 98.62 | 6.08% | 1524.6 | 0.3733 |
| Area 3 | 161.90 | 8.68% | 1703.5 | 0.3650 |
| Area 4 | 587.93 | 21.52% | 2144.5 | 0.3186 |

综上所述，舟山群岛的风光资源在季节尺度上表现出典型的负相关互补特征，潮流能的稳定出力与波浪能的冬季增强共同强化了系统的整体协同性。熵权法在权重分配上较好地平衡了利用率与稳定性，相较于最小标准差法在工程实践中更具鲁棒性与应用价值。

Discussion

本研究揭示了舟山群岛区域风、波、潮、光多种可再生能源在时间和空间尺度上的互补特征。结果表明，风能与太阳能在季节尺度上呈显著的负相关关系：冬季风能出力较高，而夏季太阳辐照最强。这种“冬风夏光”的互补模式体现出典型的季风型海洋能分布特征，为构建区域多能互补体系提供了自然基础。

Figure 2这种显著的季节互补性主要源于东亚季风系统的年内变化规律。冬季受强盛的西北季风和海陆温差作用，舟山海域风速普遍增强，风能出力显著提升；而夏季副热带高压控制下，云量减少、辐射条件改善，太阳能资源达到峰值。二者在季节尺度上的反相关性反映出气候驱动下的能量转换特征。这种规律性变化使得风光资源在时间上形成天然的错峰关系，从而有效减弱了系统输出的季节波动。

与国内外类似研究的结果相比，本研究的发现具有较高一致性。Lv 等（2022）对中国风能与太阳能的时空分布及互补性分析指出，全国风能与太阳能互补性较好的区域主要包含东南诸河流域的浙江沿海区域[41]；Wen 等（2025）的全球海上风光互补研究也显示，东亚沿海普遍存在季节负相关的互补格局[42]。本研究的结果与上述研究一致，但互补程度更为明显，可能与舟山群岛复杂的岛屿地形与多能资源叠加效应有关。此外，与北海、地中海等温带海域相比，东海沿岸的季风周期更强，风能季节振幅更大，进一步强化了风光负相关特征。这说明气候背景与区域动力学过程共同决定了多能互补性的时空差异。

在多能系统层面，波浪能与潮汐能在一定程度上弥补了风光的季节空缺。研究结果表明，波能在冬季与风能同向增强，而潮汐能在季节上相对平稳，在春秋过渡期起到削峰填谷的调节作用。这种多能耦合特征不仅提升了系统出力的稳定性，也表明海洋能在复合系统中的协同潜力。尤其是在风光互补度较低的特定月份，潮汐能的稳定输出为系统提供了基础负载支撑，具有重要的系统价值。

Area 2 & 3区域因其卓越的内禀互补性，是进行电网级大规模并网项目的理想选择。在配比上，应重点维持潮流能和风能、太阳能的稳定比例，对外部储能系统的依赖程度可以适当降低，从而节约初始投资成本。而Area 4鉴于其巨大的能源潜力但伴随高弃电风险，该区域的开发应采纳技术耦合策略，如大规模储能整合，配置大容量电池储能系统，用于吸收超额峰值电力，并将其平移至高价时段出售；能源转换即将弃电量用于电解水制氢或海水淡化，实现能源的跨领域利用，将低价的电力转化为高价值的燃料或资源。

从工程与规划的角度看，这种互补性对岛屿型能源系统具有现实意义。舟山地区风光互补特征可显著降低储能系统的容量需求：冬季由风电主导发电，夏季由光伏补充出力，实现季节性负荷的自然匹配。同时，波能与潮汐能的引入进一步提升系统的可靠性和利用率。这为未来在海岛地区推广多能协同供能模式提供了理论依据，也为政策层面制定区域能源布局方案提供了科学参考。该方法的核心优势在于其数据驱动和客观赋权的特性，使其能够适用于任何面临多源配比和波动性挑战的能源系统。韩国的加沙岛（Gasa Island）或欧洲的偏远渔业岛屿，它们同样依赖多种海洋能源，但各能源资源的时间特性差异巨大。通过输入当地的资源容量系数数据，该模型能迅速提供客观、高效的初始配比方案。

需要指出的是，本研究基于 ERA5 再分析数据进行能量互补性评估，数据空间分辨率和时间分辨率可能对结果存在一定影响；此外，模型尚未考虑储能、输电约束及经济性指标的综合作用，这可能导致系统效益存在一定偏差。未来研究将结合高分辨率观测资料与多场景优化模型，进一步探索多能系统的动态运行特性与经济可行性。

Conclusions

本研究基于 ERA5 再分析数据，构建了舟山群岛区域的风、波、潮、光多能互补性综合评价体系。通过熵权法确定各互补性指标权重，量化了多种海洋能源在时间与空间尺度上的互补特征，为海上可再生能源系统的协同开发提供了方法与理论支撑。主要结论如下：

舟山群岛区域的风能与太阳能在季节尺度上表现出显著的负相关关系：风能在冬季达到峰值，而太阳能在夏季最强，呈现典型的“冬风夏光”季风型互补特征。这种时间互补性显著降低了系统出力的季节波动性。

波浪能与潮汐能在多能体系中起到重要的平衡作用。波能在冬季随风场增强而上升，潮汐能则保持相对稳定，在过渡季节发挥削峰填谷效应。四能协同作用使系统综合稳定性显著提高，验证了海上多能互补模式的可行性与优势。

熵权法结果显示，系统波动性指标的权重最高，表明在多能协同体系中，稳定性是评价系统性能的关键因子。综合互补指数结果进一步指出，舟山北部及中部岛屿周边海域具有最高的多能互补潜力，是优先布局海上综合能源系统的理想区域。

本研究结果对岛屿型能源系统的工程规划具有参考价值。通过合理配置风光装机比例并引入波、潮能，可在不显著增加储能容量的情况下提升系统可靠性和可再生能源利用率。这为未来我国东部沿海岛屿推进“多能互补、源网储协同”的能源转型提供了科学依据。

需要指出的是，本研究未考虑储能调度、经济性约束及电网接入等实际运行条件，结果更多反映自然资源层面的互补潜力。未来工作将进一步引入储能系统模拟、成本—效益分析与多情景优化模型，以完善海上多能系统的全生命周期评估框架，实现从资源互补向运行优化的研究拓展。

- References

Melikoglu M. Current status and future of ocean energy sources: A global review[J]. Ocean Engineering, 2018, 148: 563-573.

Jusoh M A, Ibrahim M Z, Daud M Z, et al. Hydraulic power take-off concepts for wave energy conversion system: A review[J]. Energies, 2019, 12(23): 4510.

Saprykina Y, Kuznetsov S. Analysis of the variability of wave energy due to climate changes on the example of the Black Sea[J]. Energies, 2018, 11(8): 2020.Hu W, Dong Y, Zhang L, et al. Research on complementarity of multi-energy power systems: a review[J]. iEnergy, 2023, 2(4): 275-283.

Al-Ashwal A M, Moghram I S. Proportion assessment of combined PV-wind generating systems[J]. Renewable energy, 1997, 10(1): 43-51.

Papantonis D, Soldatos P, Papadakis G, et al. Evaluation of an integrated renewable energy system for electricity generation in rural areas[J]. 1997.

Li Z, Su J, Yu H, et al. Multi-Energy System Based on Ocean Thermal Energy Conversion[J]. 2020.

Quevedo A M, Domínguez E J M, de León Izquier J M, et al. Gorona del viento wind-hydro power plant[C]//3rd International Hybrid Power Systems Workshop. Energynautics GmbH, Darmstadt. 2018.

Pearre N, Swan L. Combining wind, solar, and in-stream tidal electricity generation with energy storage using a load-perturbation control strategy[J]. Energy, 2020, 203: 117898.

Wang, J., Cao, S., Li, R., & Xu, W. (2025). Energy Symbiosis in Isolated Multi-Source Complementary Microgrids: Diesel–Photovoltaic–Energy Storage Coordinated Optimization Scheduling and System Resilience Analysis. Energies, 18(21), 5741. https://doi.org/10.3390/en18215741

Parkinson S C, Dragoon K, Reikard G, et al. Integrating ocean wave energy at large-scales: A study of the US Pacific Northwest[J]. Renewable Energy, 2015, 76: 551-559.

Stoutenburg E D, Jenkins N, Jacobson M Z. Power output variations of co-located offshore wind turbines and wave energy converters in California[J]. Renewable energy, 2010, 35(12): 2781-2791.

Musial W, Heimiller D, Beiter P, et al. Offshore wind energy resource assessment for the United States[J]. National Renewable Energy Laboratory, 2016.

Khan M H A, Heywood P, Kuswara A, et al. An integrated framework of open-source tools for designing and evaluating green hydrogen production opportunities[J]. Communications Earth & Environment, 2022, 3(1): 309.

Psarros G N, Dratsas P A, Chinaris P P, et al. Assessing the implications of RES technology mix on curtailments, storage requirements and system economics[J]. Applied Energy, 2025, 381: 125159.

Vázquez R, Cabos W, Nieto-Borge J C, et al. Complementarity of offshore energy resources on the Spanish coasts: Wind, wave, and photovoltaic energy[J]. Renewable Energy, 2024, 224: 120213.

Shi J, Zhang Y, Kong D, et al. A review on research and application of multi-energy complementary system containing high proportion of renewable energy based on different attributes[J]. Archives of Thermodynamics, 2023: 3-41-3-41.

Sawle Y, Gupta S C, Bohre A K. Optimal sizing of standalone PV/Wind/Biomass hybrid energy system using GA and PSO optimization technique[J]. Energy Procedia, 2017, 117: 690-698.

Abo-Elyousr F K, Elnozahy A. Bi-objective economic feasibility of hybrid micro-grid systems with multiple fuel options for islanded areas in Egypt[J]. Renewable Energy, 2018, 128: 37-56.

Lu J, Wang W, Zhang Y, et al. Multi-objective optimal design of stand-alone hybrid energy system using entropy weight method based on HOMER[J]. Energies, 2017, 10(10): 1664.

Lu Y, Chen Z, Duan Q, et al. Evaluation of smart energy system based on AHP-improved entropy weight method-TOPSIS[J]. Smart Grid, 2020, 10(1): 21-28.

Shuyi H, Xuetong Z, Huaguang L, et al. Comprehensive evaluation of multi-energy complementary ecosystem based on improved multicriteria decision-making method[J]. Clean Energy, 2024, 8(1): 226-236.

Wan Y, Fan C, Zhang J, et al. Wave energy resource assessment off the coast of China around the Zhoushan Islands. Energies 10 (9): 1320[EB/OL].(2017)

Tang D, Xiao Z, Li J, et al. Assessment of coastal wind energy resources in Ningbo Zhoushan Port Area based on WRF model and MERRA-2 data[C]//2023 7th International Conference on Transportation Information and Safety (ICTIS). IEEE, 2023: 19-24.

Lin Y, Dong S, Wang Z, et al. Wave energy assessment in the China adjacent seas on the basis of a 20-year SWAN simulation with unstructured grids[J]. Renewable Energy, 2019, 136: 275-295.

Ye Z, Gu W, Ji Q. Study on Critical Factors Affecting Tidal Current Energy Exploitation in the Guishan Channel Area of Zhoushan[J]. Sustainability, 2022, 14(24): 16820.

Wen Y, Kamranzad B, Lin P. Joint exploitation potential of offshore wind and wave energy along the south and southeast coasts of China[J]. Energy, 2022, 249: 123710.

Pecher A, Kofoed J P, Espedal J, et al. Results of an experimental study of the Langlee wave energy converter[C]//ISOPE International Ocean and Polar Engineering Conference. ISOPE, 2010: ISOPE-I-10-048.

Elibol E, Özmen Ö T, Tutkun N, et al. Outdoor performance analysis of different PV panel types[J]. Renewable and Sustainable Energy Reviews, 2017, 67: 651-661.

Almoghayer M A, Woolf D K, Kerr S, et al. Integration of tidal energy into an island energy system–A case study of Orkney islands[J]. Energy, 2022, 242: 122547.

Wen Y, Kamranzad B, Lin P. Assessment of long-term offshore wind energy potential in the south and southeast coasts of China based on a 55-year dataset[J]. Energy, 2021, 224: 120225.

Kamranzad B, Lin P, Iglesias G. Combining methodologies on the impact of inter and intra-annual variation of wave energy on selection of suitable location and technology[J]. Renewable Energy, 2021, 172: 697-713.

Almoghayer M A, Woolf D K, Kerr S, et al. Integration of tidal energy into an island energy system–A case study of Orkney islands[J]. Energy, 2022, 242: 122547.

Jerez S, Tobin I, Vautard R, et al. The impact of climate change on photovoltaic power generation in Europe[J]. Nature communications, 2015, 6(1): 10014.

方舣洲, 武贺. 舟山群岛海域主要水道潮流能资源评估方法与分析[J]. 地理信息世界, 2022.

樊超, 刘艳萍. 舟山群岛太阳能资源开发管理区域分析[J]. 广东化工, 2015, 42(17): 95-96.

Tang D, Xiao Z, Li J, et al. Assessment of coastal wind energy resources in Ningbo Zhoushan Port Area based on WRF model and MERRA-2 data[C]//2023 7th International Conference on Transportation Information and Safety (ICTIS). IEEE, 2023: 19-24.

Wan Y, Fan C, Zhang J, et al. Wave energy resource assessment off the coast of China around the Zhoushan Islands. Energies 10 (9): 1320[EB/OL].(2017)

Zhao B, Chen J, Zhang L, et al. Three representative island microgrids in the East China Sea: Key technologies and experiences[J]. Renewable and Sustainable Energy Reviews, 2018, 96: 262-274.

Husein M, Hau V B, Chung I Y, et al. Design and dynamic performance analysis of a stand-alone microgrid-A case study of Gasa Island, South Korea[J]. Journal of Electrical Engineering and Technology, 2017, 12(5): 1777-1788.

Bertheau P. Supplying not electrified islands with 100% renewable energy based micro grids: A geospatial and techno-economic analysis for the Philippines[J]. Energy, 2020, 202: 117670.

https://henb.nea.gov.cn/hdhy/zlxz/202508/t20250805_285315.html

Psarros G N, Dratsas P A, Chinaris P P, et al. Assessing the implications of RES technology mix on curtailments, storage requirements and system economics[J]. Applied Energy, 2025, 381: 125159.

https://www.zhoushan.gov.cn/art/2025/2/15/art_1229029577_59294224.html

Yang J, Chi H, Cheng M, et al. Performance analysis of hydrogen supply using curtailed power from a solar-wind-storage power system[J]. Renewable Energy, 2023, 212: 1005-1019.

Lv A, Li T, Zhang W, et al. Spatiotemporal distribution and complementarity of wind and solar energy in China[J]. Energies, 2022, 15(19): 7365.

Wen Y, Wu J, Lin P, et al. The role of offshore wind and solar PV resources in global low-carbon transition[J]. Science Advances, 2025, 11( 43): eadx5580.